Worksheet 2: Working with polymers

Task 1 Stock forms types and sizes

Calculate suitable the sizes and forms of material required to make the desk tidy shown. Justify suitable thicknesses appropriate for use.

Task 2

Name the type of screw heads pictured below. Sketch out two other different screw heads that you discover.

Look at the images in the table below. Fill in the missing boxes to suggest which hinge types would be most suitable for each use and justify your reasons.

| Image | Hinge type | Justification |
| --- | --- | --- |
| Acrylic screen on an advertising billboard |  |  |
| Tool box |  |  |
| Cosmetics case |  |  |

Task 3

Featured below are three of saws used to cut plastic.

Name each saw. Explain one feature of, or advantage to using each saw 
for cutting plastic.

| Name | Saw | Advantages |
| --- | --- | --- |

Task 4

This display stand requires line bending. Put the stages listed for the process of making it in the correct order. The first one has been done for you.

| Correct order | Stage |
| --- | --- |
| 1 | Use a chinagraph pencil to mark out where the bend lines should be |
|  | Place the marked line of the workpiece across the heating strip |
|  | Turn on the strip heater so that it comes up to a working temperature |
|  | Allow the plastic to heat through |
|  | Bend the workpiece to the required angle and repeat for other bends |
|  | Once the workpiece has set it can be cooled in the water tray |
|  | Put on heat-proof gloves and have a tray of water ready to cool the workpiece |
|  | Test for flexibility as the workpiece approaches the right temperature |

What two safety precautions could you take when line bending?

Task 5

When vacuum forming, a mould should conform to various properties to ensure there are no issues covered by points A, B and C below.

- To ensure easy removal of the mould

- To ensure the plastic is not drawn too thin and breaks

- To avoid air pockets

State which of A, B or C, each of the properties below aim to resolve.

- A positive draft angle of at least 3° –

- No undercuts –

- A profile that is not too deep –

- Vent holes drilled –

- Corners and edges rounded with a small radius –

- A smooth finish and a release agent can be applied to the mould –

Task 6

Describe how 3D printed objects are created using fused deposit modelling (FDM)?

What advantages are there to 3D printing a prosthetic arm?